Technical Specification for UUTP CAT6 4PR 23AWG PVC CPA Conductor

## 1. General Information

### 1.1 Cable Type

U/UTP CAT 6

### 1.2 Impedance

100 ohm

### 1.3 Length Options

100m, 305m, 500m, 500ft, 1000ft

### 1.4 Packing Options

Pull box, wooden spool, carton, pallet

### 1.5 Minimum Order Quantity (MOQ)

30KM

### 1.6 Port of Loading

SHANGHAI

2.1 Conductor

• Material: Solid CPA (Copper Plating Aluminum)

• Size: 23AWG

2.2 Pairs

4 Pairs

2.3 Insulation

• Thickness: 1.02±0.05 mm HDPE

• Material: HDPE

• Colors: Blue-White/Blue, Orange-White/Orange, Green-White/Green, Brown-White/Brown

2.4 Rip Cord

Cotton

2.5 Outer Jacket

• Material: PVC

• Wall Thickness: 0.0157" Nom(0.4mm)

• Outer Diameter (OD): 6.4±0.2mm

3. Standards Compliance

UL 444 & CSA C22.2 No.214 Communications Cables

4. Application Support

• 1000BASE-T, 1000BASE-TX, 100BASE-T, 100BASE-TX, 100VG-AnyLAN, ATM

• VoIP, ISDN, Token, 100MTPPDM, Analog and Data video, POE Type1,2,3,4

5. Electrical Characteristics

| Impedance | 1 - 250MHz 100±15 Ohms |
| --- | --- |
| Insulation Resistance | ＞5000MΩ*KM |
| Dielectric strength | DC 2500V 2S |
| Max.Conductor DC | 10.8 Ohms/100m |
| Resistance 20℃ |  |
| Res. unbalance (%) | Max. 2.5 |
| Pair to Ground | 330 pF/100m |
| Capacitance Unbalance |  |
| Transfer Impendance | Max 10 mOhms/m @100Mhz |
| Installation temp. | -30 - 50℃ |
| Operation temp. | -20 - 75℃ |

7. Physical Properties

| Parameter | Specification |
| --- | --- |
| Pulling Tension (Max) | 25 lbs(110N) |
| Bend Radius (Min) | 4 Times the cable diameter |
| Operating Temperature | -20 to 75 ℃ （ -4 to 167 ℉） |