Technical Specification for Rack Mount Type PLC Splitter

## 1. Overview

The Rack Mount Type PLC (Planar Lightwave Circuit) Splitter is a fully passive optical branching device designed for optical signal power splitting. It features excellent performance characteristics such as low insertion loss, low Polarization Dependent Loss (PDL), high channel uniformity, and a wide operating wavelength range. This splitter is engineered to meet the rigorous requirements of various optical network applications.

## 2. Applications

- FTTH (Fiber to the Home), FTTC (Fiber to the Curb), FTTB (Fiber to the Building)

- Passive Optical Networks (PON)

- CATV networks and data communications systems

- Fiber optic equipment and related systems

## 3. Key Features

- Complies with Telcordia-1209-core and GR-1221-core standards for high performance

- High channel uniformity ensuring consistent signal distribution

- Low insertion loss minimizing signal attenuation

- Compact and small size for space-efficient installation

- High reliability and stability in diverse operating conditions

- Optional input and output configurations available

- Utilizes Single Mode Fiber Type G657A

## 4. Technical Specifications

| Parameter | Specification |
| --- | --- |
| Operating Wavelength Range | 1260 - 1650 nm |
| Insertion Loss (with connector) | 1×4: ≤7.6 dB<br>1×8: ≤10.5 dB<br>1×16: ≤13.8 dB<br>1×32: ≤17.1 dB<br>1×64: ≤20.6 dB<br>2×4: ≤7.5 dB<br>2×8: ≤10.8 dB<br>2×16: ≤14.1 dB<br>2×32: ≤17.4 dB |
| Polarization Dependent Loss (PDL) | 1×4: ≤0.2 dB<br>1×8: ≤0.19 dB<br>1×16: ≤0.18 dB<br>1×32: ≤0.15 dB<br>1×64: ≤0.4 dB<br>2×4: ≤0.2 dB<br>2×8: ≤0.2 dB<br>2×16: ≤0.19 dB<br>2×32: ≤0.18 dB |
| Physical Size (H×W×L) | 45×205×483 mm |
| Return Loss | UPC: ≥50 dB<br>APC: ≥55 dB |
| Directivity | ≥55 dB |
| Operating Temperature Range | -40 ~ +85 °C |