Technical Specification for 6/8/12 Cores G.652.D Fiber ADSS Cable

## 1. Product Overview

All-dielectric self-supporting (ADSS) cable with 6/8/12 cores of G.652.D single-mode optical fiber, featuring a double jacket PE sheath. Designed for outdoor use with excellent mechanical strength, environmental resistance, and optical performance, suitable for long-distance signal transmission in various climates.

## 2. Key Parameters

- Max Span: 200-500m

- Max Applied Voltage: 110kv

- Weather Resistance: 25m/s wind speed, 0mm ice load

- Temperature Range: -40°C to +70°C (operating); -20°C to +60°C (installation); -40°C to +70°C (transport & storage)

- Length per Spool: 3km or 4km (±2% tolerance)

- Cable Outer Diameter: 10.7±0.5mm

- Cable Weight: 85±5kg/km

## 3. Construction Details

| Component | Specification |
| --- | --- |
| Optical Fiber | G.652.D UV fiber; Attenuation: ≤0.35dB/km @1310nm, ≤0.22dB/km @1550nm |
| Tube Filling Compound | Water-blocking & moisture-proof thixotropic gel |
| Loose Tube | PBT material, 1.8-2.0mm outer diameter |
| Filler | FRP (1.8mm), LDPE/PP material |
| Central Strength Member | GFRP (1.8mm), glass fiber reinforced plastic rod |
| Water Blocking Tape | 0.28mm thickness |
| Binder | 2 wires of polyester yarns |
| Ripcords | 2 wires |
| Dielectric Strength Member | Aramid Yarns (KEVLAR K49-3000D) |
| Inner Jacket | HDPE/MDPE, 0.9-1.0mm thickness |
| Outer Jacket | HDPE/MDPE, 1.6-2.0mm thickness |

## 4. Optical Characteristics

### 4.1 Fiber Types & Attenuation

| Fiber Type | Attenuation (dB/km) @850nm | @1300nm | @1310nm | @1550nm |
| --- | --- | --- | --- | --- |
| G.652.D (SM) | - | - | ≤0.36 | ≤0.22 |
| G.655 (SM) | - | - | ≤0.40 | ≤0.23 |
| 50/125μm (MM) | ≤3.0 | ≤1.0 | - | - |
| 62.5/125μm (MM) | ≤3.0 | ≤1.0 | - | - |

### 4.2 Bandwidth (Multimode)

- 50/125μm: ≥500 MHz·km @850nm, ≥1000 MHz·km @1300nm

- 62.5/125μm: ≥200 MHz·km @850nm, ≥600 MHz·km @1300nm

### 4.3 Polarization Mode Dispersion (PMD)

- Individual fiber: ≤0.20 ps/√km (G.652.D, G.655)

- Design Link Value (M=20, Q=0.01%): ≤0.10 ps/√km (G.652.D, G.655)

### 4.4 Transmission Bands

| Band | Wavelength Range (nm) | Frequency Range (THz) | Application |
| --- | --- | --- | --- |
| O Band | 1260-1360 | 237.9-220.4 | Multimode systems |
| E Band | 1360-1460 | - | Less common (high attenuation) |
| S Band | 1460-1530 | 205.3-195.9 | Passive Optical Networks (PON) |
| C Band | 1530-1565 | 195.9-191.6 | Long-haul transmission (min attenuation) |
| L Band | 1565-1625 | 191.6-184.5 | Extended reach, high-capacity networks |
| U Band | 1625-1675 | 184.5-179.0 | Network monitoring |

### 4.5 Additional Optical Parameters

- Mode Field Diameter (MFD): Compliant with ITU-T Rec. G.652/G.657

- Point Discontinuity: ≤0.05 dB @1310nm & 1550nm (all fibers)

## 5. Dimensional Specifications

| Parameter | G.652.D Normal | G.652.D Best | G.657.A1 | G.657.A2 |
| --- | --- | --- | --- | --- |
| Cladding Diameter (μm) | 125±0.7 | 125±0.5 | 125±0.5 | 125±0.5 |
| Core/Cladding Concentricity Error (μm) | ≤0.5 | ≤0.4 | ≤0.4 | ≤0.4 |
| Cladding Non-Circularity (%) | ≤1.0 | ≤0.7 | ≤0.7 | ≤0.7 |
| Coating Diameter (μm) | 245±5 | 245±5 | 242±5 | 242±5 |
| Coating/Cladding Concentricity Error (μm) | ≤12 | ≤12 | ≤8 | ≤8 |

## 6. Mechanical Characteristics

- Fiber Strain: ≥1%

- Fiber Load: ≥9 N

- Stress Resistance: ≥100 kpsi

- Dynamic Stress Corrosion Susceptibility Factor: ≥20 (unaged & aged: 30 days @85°C, 85% R.H.)

- Coating Strip Force (Peak Value): 1.3-8.9 N

- Fiber Curl: ≥4 m

- Max Installation Tension: 2400 N

- Min Bending Radius: 10×cable diameter (without tension); 20×cable diameter (under max tension)

## 7. Environmental Characteristics

| Test Condition | Attenuation Change (dB/km) @1310nm | @1550nm |
| --- | --- | --- |
| Dry Heat Aging (30 days @85°C) | ≤0.05 | ≤0.05 |
| Accelerated Aging (30 days @85°C, 85% R.H.) | ≤0.05 | ≤0.05 |
| Temperature Cycling | ≤0.05 | ≤0.05 |
| Water Soak (30 days @23°C) | ≤0.05 | ≤0.05 |

## 8. Mechanical & Environmental Tests (Compliance Standards)

| Test Item | Method | Acceptance Criteria |
| --- | --- | --- |
| Tensile Strength | IEC 794-1-E1; 5000N load on 50m cable | ≤0.1dB loss @1550nm; no fiber break/sheath damage |
| Crush Test | IEC 60794-1-E3; 3000N load on 100mm cable (≥1min) | Same as above |
| Impact Test | IEC 60794-1-E4; 5 points×5 impacts, 4.5Nm energy | Same as above |
| Repeated Bending | IEC 60794-1-E6; 20×OD bend, 250N load, 30 cycles | Same as above |
| Torsion | IEC 60794-1-E7; 1m length, 150N load, ±90° twist (10 cycles) | Same as above |
| Water Penetration | IEC 60794-1-F5B; 3m sample in 1m water (24h) | No water leakage |
| Temperature Cycling | IEC 60794-1-F1; +20→-20→+20→+40°C (2 cycles, 24h/step) | Same as tensile strength |
| Compound Flow | IEC 60794-1-E14; 30cm sample @70±2°C (24h) | No compound flow |
| Sheath High Voltage Test | Online test (9kV, dependent on sheath thickness) | No sheath breakdown |

## 9. Standards Compliance

- ITU-T Rec. G.657A, G.652.D

- ISO9001

- IEC 60794

- ICEA-596

- GR-409

- YD/T 1997-2009

- EIA/TIA 598B (fiber/binder color code)

## 10. Fiber & Cable Marking

- Fiber Colors (12 chromatography): Blue, Orange, Green, Brown, Gray, White, Red, Black, Yellow, Violet, Aqua, Pink

- Cable Marking: Printed every 1m, including cable type, core count, G.652D specification, manufacture year, and length. Color: white. Both ends sealed with heat-shrinkable caps.

## 11. Packing Specifications

- Spool Type: Wooden drum (Φ1100×750mm)

- Packing: Cable reeled on wooden drum, covered with plastic buffer sheet, sealed with wooden battens.

- Reserved Length: ≥1m inside end for testing.

- Drum Marking: Includes cable type, size, length, gross/net weight, manufacture year/month, and handling cautions. "MADE IN CHINA" marked.

## 12. Customization (OEM)

- Core Count Options: 12,24,48,72,96,144,288 fibers

- Loose Tube Configuration: Customizable (e.g., 1x6, 2x6 for 6 fibers; 4x12, 6x12 for 12 fibers)

- Outer Diameter Adjustment: 8.0-11.0mm (customizable)

- Central Strength Member: Optional FRP/steel wire (diameter: 1.0-3.7mm)

- Outer Sheath Thickness: 0.8-2.0mm (dependent on core count)