Technical Specification for PLC Splitter

## 1. Material List

| No. | Material Name | Quantity |
| --- | --- | --- |
| 1 | PLC Splitter | - |
| 2 | Plastic Plug-in Box (Dimensions: 877224.5mm) | 1 |
| 3 | SC/APC Fiber Adapter | 4/8 |

## 2. Performance Requirements (Fiber Type: G652D)

| Splitter Type | Performance Parameters |
| --- | --- |
| 1x4 | Insertion Loss (IL): ≤7.7dB (With Connector); ≤7.5dB (Without Connector)<br>Polarization Dependent Loss (PDL): ≤0.2dB<br>Uniformity: ≤0.8dB<br>Directivity: ≥55dB<br>Return Loss (RL): ≥55dB |
| 1x8 | Insertion Loss (IL): ≤10.7dB (With Connector); ≤10.5dB (Without Connector)<br>Polarization Dependent Loss (PDL): ≤0.2dB<br>Uniformity: ≤0.8dB |

## 3. Applications

- FTTH (Fiber to the Home), FTTC (Fiber to the Curb), FTTB (Fiber to the Building)

- Passive Optical Networks (PON)

- CATV networks and data communications systems

- Fiber optic equipment and related systems integration