Technical Specification for Outdoor SFUTP CAT6 23AWG PE Ethernet Cable

## 1. General Information

### 1.1 Cable Type

SF/UTP CAT 6

### 1.2 Impedance

100 ohm

### 1.3 Certifications

UL, ETL, CE, CPR, RoHS

### 1.4 Length Options

100m, 305m, 500m, 500ft, 1000ft

### 1.5 Packing Options

Pull box, wooden spool, carton, pallet

### 1.6 Minimum Order Quantity (MOQ)

30KM

### 1.7 Delivery Time

Normally 25 working days (25 days after receiving the deposit)

### 1.8 Port of Loading

NINGBO, SHANGHAI

### 1.9 Payment Terms

T/T, L/C at sight, D/P at sight

2.1 Conductor

• Material: Solid Bare Copper

• Size: 23AWG

2.2 Pairs

4 Pairs

2.3 Insulation

• Thickness: 1.14±0.05 mm HDPE

• Material: HDPE

• Colors: Blue-White/Blue, Orange-White/Orange, Green-White/Green, Brown-White/Brown

2.4 Rip Cord

Cotton

2.5 Outer Jacket

• Material: PE

• Wall Thickness: 0.0157" Nom(0.4mm)

• Outer Diameter (OD): 8.1±0.2mm

3. Standards Compliance

UL 444 & CSA C22.2 No.214 Communications Cables

4. Application Support

• 1000BASE-T, 1000BASE-TX, 100BASE-T, 100BASE-TX, 100VG-AnyLAN, ATM

• VoIP, ISDN, Token, 100MTPPDM, Analog and Data video, POE Type1,2,3,4

5. Electrical Characteristics

| Parameter | Specification |
| --- | --- |
| DC Resistance | ＜ 9.38 Ohms/100m @20 ℃ |
| DC Resistance Unbalance | 5% @ 20℃ |
| Mutual Capacitance | 5.6 nF/100m |
| Characteristic Impedance | 1 - 100 MHz : 100±15 Ohms; 101 - 250 MHz : 100±22 Ohms |
| Max Frequency | 250MHz |
| Capacitance Unbalance | 330 pF/100m |
| NVP (Nominal Velocity of Propagation) | ≥ 65% |
| Delay Skew | ≤ 44ns/100m |
| Working Voltage | Typically up to 48V |
| Rated Voltage (V RMS) | 300V |

6. Transmission Performance (100m, 20±2℃)

| Frequency (MHz) | RL (Min.) dB | Insertion Loss (Max.) dB/100m | Propagation Delay (Max.) ns/100m | Delay Skew (Max.) ns/100m | NEXT (Min.) dB | PSNEXT (Min.) dB | ELNEXT (Min.) dB/100m | PSELNEXT (Min.) dB/100m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 20 | 2 | 570 | 45 | 74 | 72 | 67.8 | 64.8 |
| 4 | 23 | 3.8 | 552 | 45 | 65 | 63 | 55.7 | 52.7 |
| 10 | 25 | 6 | 545 | 45 | 59 | 57 | 47.8 | 44.8 |
| 16 | 25 | 7.6 | 543 | 45 | 56 | 54 | 43.7 | 40.7 |
| 20 | 25 | 8.5 | 542 | 45 | 55 | 53 | 41.7 | 38.7 |
| 31.25 | 23.6 | 10.7 | 540 | 45 | 52 | 50 | 37.9 | 34.9 |
| 62.5 | 21.5 | 15.4 | 539 | 45 | 47 | 45 | 31.8 | 28.8 |
| 100 | 20.1 | 19.8 | 538 | 45 | 44.3 | 42.3 | 27.8 | 24.8 |
| 200 | 18 | 29 | 537 | 45 | 39.7 | 37.7 | 21.7 | 18.7 |
| 250 | 17.3 | 32.8 | 536 | 45 | 38 | 36 | 19.8 | 16.8 |

7. Physical Properties

| Parameter | Specification |
| --- | --- |
| Pulling Tension (Max) | 25 lbs(110N) |
| Bend Radius (Min) | 4 Times the cable diameter |
| Operating Temperature | -20 to 75 ℃ （ -4 to 167 ℉） |